Design Laboratory

Documentation for a setup of an automated test environment in NI TestStand

Project completed by: Artem Kmet, Adam Kruszec

Instructor: Msc Eng. Sebstian Koryciak

29.01.2026r.

Introduction:

All test sequences were developed within the NI TestStand application. As part of the National Instruments ecosystem, TestStand allows for the integration of pre-configured Python scripts to automate sequences, provide a user-friendly GUI, and generate comprehensive test reports.

Task 1: Oscilloscope and Function Generator Verification

In order to ensure that the oscilloscope is working correctly, we measure the signal at different voltage values on a channel selected by the user – between the minimum and maximum values, with a specified step and a specified number of times. All configuration values must be specified by the user. A generator is used to set the DC voltage. Also, at the beginning of the sequence if multiple devices are connected user can choose which to use by inputting UUT (Unit under test) number.

Task 2: Switch Input/Output Configuration

First, an attempt is made to connect to the server with the IP address and port specified by the user. After a successful connection, the user selects which outputs will be assigned to each input. It should be noted that two inputs cannot be connected to the same output.

Task 3: IPbus Operational Testing

The procedure begins with an attempt to connect to the IPbus module at the IP address and port specified by the user. Next, the temperature is measured and the presence of a connected attenuator is verified. In the final stage, the output voltage is measured at variable input signal values (ranging from minimum to maximum, with a specified step and a set number of repetitions). All configuration parameters are defined by the user, and measurements are performed on an ADC.